Classification by product type (Plastic insulated）　　　　　　　  　　                  　Cables

| Factor |  | Classification |  |
| --- | --- | --- | --- |
| A | Conductor main materials | 1 | Copper |
|  |  | 2 | Others |
| B | Conductor size | 1 | Stranded conductor and shaped solid conductor with cross section of 8.0㎟ or less Solid conductor (excluding shaped solid conductor) with diameter of 3.2mm or less |
|  |  | 2 | Stranded conductor and shaped solid conductor with cross section of exceeding 8.0㎟ and less than 32㎟ Solid conductor (excluding shaped solid conductor) with diameter of exceeding 3.2mm |
|  |  | 3 | Stranded conductor and shaped solid conductor with cross section of exceeding 32㎟ |
| C | Insulation main materials | 1 | Polyvinyl chloride mixture |
|  |  | 2 | Heat resistant polyvinyl chloride mixture |
|  |  | 3 | Polyethylene mixture |
|  |  | 4 | flame resistant polyethylene mixture |
|  |  | 5 | cross linked polyethylene mixture |
|  |  | 6 | flame resistant cross linked polyethylene mixture |
|  |  | 7 | Silicon rubber mixture |
|  |  | 8 | Heat resistant silicon rubber mixture |
|  |  | 9 | Other mixtures |
| D | Sheath main materials | 1 | Chloroprene rubber mixture |
|  |  | 2 | Chlorosulfonated polyethylene rubber mixture |
|  |  | 3 | Silicon rubber mixture(Limited to those with enhanced mechanical strength) |
|  |  | 4 | Polyvinyl chloride mixture |
|  |  | 5 | Heat resistant polyvinyl chloride mixture |
|  |  | 6 | Polyethylene mixture |
|  |  | 7 | flame resistant polyethylene mixture |
|  |  | 8 | cross linked polyethylene mixture |
|  |  | 9 | flame resistant cross linked polyethylene mixture |
|  |  | 10 | Other mixtures |
| E | Core | 1 | Single core |
|  |  | 2 | Two or more cores |
| Ｆ | Applications (limited to Polyvinyl chloride mixture sheathed cables including heat resistant ones) | 1 | For concrete buried cable conductor |
|  |  | 2 | Others |
| G | Applications (limited to heat resistant polyethylene sheathed cables) | 1 | For access floor wiring system |
|  |  | 2 | Others |

Note：Please mark the factor symbol and classification number　　　　　　　　　　　　                 GS02 -rev